### E1: Pipeline Speedup Analysis

Question:
A non-pipelined processor operates with a 10ns clock cycle, and for a specific workload, instructions average 4.4 clock cycles. In contrast, a pipelined variant introduces a 1ns pipeline overhead due to clock skew, with each of its 5 stages taking 10ns. Assuming the workload can be perfectly divided across increasing pipeline stages (from 5 to 20), calculate and plot the performance gain of pipelining compared to the non-pipelined case. Also, plot the ideal speedup ignoring the overhead.

Answer:
Using the CPU performance equation:
CPU Time = Instruction Count × CPI × Clock Cycle Time

Given:

- Non-pipelined clock = 10ns

- CPI (non-pipelined) = 4.4

- CPI (pipelined) = 1

- Total instruction time = 50ns (5 stages × 10ns)

- Pipeline clock = 50nsN+1ns\frac{50ns}{N} + 1nsN50ns​+1ns, where N is the number of stages.

Ideal Speedup:
- 5 stages: 4.4
- 6 stages: 5.28
- 10 stages: 8.8
- 15 stages: 13.2
- 20 stages: 17.6

Actual Speedup:
- 5 stages: 4 (91% of ideal)
- 6 stages: 4.7 (89% of ideal)
- 10 stages: 7.3 (82% of ideal)
- 15 stages: 10.2 (77% of ideal)
- 20 stages: 12.6 (72% of ideal)

Ideal speedup (ignoring the 1ns overhead):

44ns50nsN=0.88×N\frac{44ns}{\frac{50ns}{N}} = 0.88 × NN50ns​44ns​=0.88×N

Calculations:

| Stages | Ideal Speedup | Actual Speedup | % of Ideal |
| --- | --- | --- | --- |
| 5 | 4.4 | 4.0 | 91% |
| 6 | 5.28 | 4.7 | 89% |
| 10 | 8.8 | 7.3 | 82% |
| 15 | 13.2 | 10.2 | 77% |
| 20 | 17.6 | 12.6 | 72% |

This illustrates Amdahl's Law, where diminishing returns are due to the fixed overhead that limits further improvements.

### E2: Loop Scheduling on a RISC Pipeline

Given Loop:

arduino

CopyEdit

loop:   LD      R1,0(R2)

DADDI   R1,R1,1

SD      0(R2),R1

DADDI   R2,R2,4

DSUB    R4,R3,R2

BNEZ    R4,loop

Assume initial R3 = R2 + 396, and the pipeline is a classic 5-stage integer pipeline (IF, ID, EX, MEM, WB), with branches resolved in ID. Each memory access takes 1 cycle.

a. Without Forwarding, Branch Prediction, or Delay Slots:
Question: How many cycles does the loop require if no forwarding is used, and pipeline flushing handles branches?

Answer:

- Pipeline stalls for data dependencies.

- Each loop iteration takes 15 cycles, except the last, which takes 18.

- 99 iterations in total (396 / 4).

- Total cycles = (98 × 15) + 18 = 1488 cycles.

b. With Forwarding and Predict-Not-Taken Branch Handling:
Question: How many cycles does the loop require with standard data forwarding and assume branches are predicted not taken?

Answer:

- Data hazards are handled by forwarding.

- Branch mispredictions still cause one stall.

- Each iteration now takes 9 cycles, final iteration 12.

- Total cycles = (98 × 9) + 12 = 894 cycles.

c. Instruction Scheduling with Delayed Branch and Forwarding:
Question: With delayed branching and forwarding, reorder instructions and fill the branch delay slot appropriately. How many cycles does the loop require?

Answer:
Instruction reordering:

arduino

CopyEdit

LD      R1,0(R2)

DADDI   R2,R2,4

DSUB    R4,R3,R2

DADDI   R1,R1,1

BNEZ    R4,loop

SD      -4(R2),R1  ; moved to branch delay slot

- Dependencies are spread out, minimizing stalls.

- Loop takes 6 cycles per iteration, final one takes 10.

- Total = (98 × 6) + 10 = 598 cycles.