### E1: Cache Design and Performance Analysis

Scenario:
You’re working with a system where the average CPI is 1.0 when all instruction and data accesses are cache hits. The architecture includes a 256 KB unified cache (serving both instructions and data), with a 4-way set associativity and a block size of 64 bytes. Half of all instructions are memory-related (loads/stores). The miss rate of the cache is 2%, and each cache miss leads to a 25-cycle delay. Both data and instruction addresses are 32 bits wide.

#### a. Determine the Cache Tag Size

- Block offset bits: log2(64) = 6 bits

- Number of sets: 256 KB / (64 bytes * 4) = 1024 sets

- Index bits: log2(1024) = 10 bits

- Tag bits: 32 - (10 + 6) = 16 bits

#### b. Performance Improvement with Perfect Cache Hit Rate

- Effective CPI (with misses):

  - Memory operations per instruction = 1 (instruction) + 0.5 (data) = 1.5

  - Stall cycles per instruction = 1.5 * 0.02 * 25 = 0.75

  - Total CPI = 1.0 (base) + 0.75 = 1.75

Conclusion:
A CPU that experiences no cache misses operates 1.75 times faster than one with the given miss rate.

### E2: Cache Bandwidth Utilization

Given System Characteristics:

- 95% of memory requests hit the cache.

- Each block includes 2 words.

- Memory and processor communicate at 10⁹ words/sec.

- 25% of accesses are writes.

- Memory system supports 10⁹ words/sec bandwidth (either read or write).

- Cache employs write-allocate on a write miss.

- The system bus handles 1 word per transaction.

- 30% of cache blocks are dirty.

#### a. Write-Through Cache

Memory traffic breakdown:

- Read hit: no memory access.

- Read miss: read 2 words from memory.

- Write hit: write 1 word to memory.

- Write miss: read 2 words + write 1 word (total = 3 words).

Average traffic:

= 0.7125*0 + 0.0375*2 + 0.2375*1 + 0.0125*3

= 0.35 words per access

Bandwidth usage: 0.35 * 10⁹ → 35% of available bandwidth

#### b. Write-Back Cache

- Read hit/write hit: no memory access

- Read miss:

  - If dirty block replaced: write 2 + read 2 = 4 words

  - If clean: only read 2 words

- Write miss:

  - If dirty block replaced: write 2 + read 2 = 4 words

  - If clean: only read 2 words

Average words transferred:

= 0.0375*(0.7*2 + 0.3*4) + 0.0125*(0.7*2 + 0.3*4)

= 0.13 words per access

Bandwidth usage: 0.13 * 10⁹ → 13% of available bandwidth

Conclusion:
Write-through consumes almost triple the bandwidth of write-back caching.

### E3: Write-Through vs Write-Back Cache – Performance Impact

Setup:

- Cache read hit: 1 cycle

- Write hit: 2 cycles

- Cache miss penalty: 50 cycles

- Dirty block write (write-back): 50 cycles

- Instruction miss rate: 0.5%

- Data miss rate: 1%

- Instruction mix: 26% loads, 9% stores

- 50% dirty blocks (write-back cache)

- Write-through cache uses a buffer (no CPU stall)

Execution CPI:

= (0.26 * 1) + (0.09 * 2) + (0.65 * 1)

= 1.09

#### Write-Through Cache:

Stall = (0.005 * 50) + 0.01 * (0.26*50 + 0.09*50)

= 0.25 + 0.175 = 0.425

Total CPI = 1.09 + 0.425 = 1.515

#### Write-Back Cache:

Stall = (0.005 * 50) + 0.01 * (0.26*(0.5*50 + 0.5*100) + 0.09*(0.5*50 + 0.5*100))

= 0.25 + 0.2625 = 0.5125

Total CPI = 1.09 + 0.5125 = 1.6025