# E1: CAR

Problem

Consider a computer system with a first-level data cache with the following characteris- tics: size: 16KBytes; associativity: direct-mapped; line size: 64Bytes; addressing: physical.

The system has a separate instruction cache and you can ignore instruction misses in this problem. This system is used to run the following code:

for (i=0; i<4096; i++) X[i] = X[i] * Y[i] + C

Assume that both X and Y have 4096 elements, each consisting of 4 bytes (single precision floating point). These arrays are allocated consecutively in physical memory. The assembly code generated by a naive compiler is the following:

loop: lw f2, 0(r1)	# load X[i] lw f4, 0(r2)	# load Y[i]

multd f2, f2, f4	# perform the multiplication addd f2, f2, f0	# add C (in f0)

sw 0(r1), f2	# store the new value of X[i] addi r1, r1, 4	# update address of X

addi r2, r2, 4	# update address of Y addi r3, r3, 1	# increment loop counter bne r3, 4096, loop # branch back if not done

- How many data cache misses will this code generate? Breakdown your answer into the three types of misses. What is the data cache miss rate?

- Provide a software solution that significantly reduces the number of data cache misses. How many data cache misses will your code generate? Breakdown the cache misses into the three types of misses. What is the data cache miss rate?

- Provide a hardware solution that significantly reduces the number of data cache misses. You are free to alter the cache organization and/or the processor. How many data cache misses will your code generate? Breakdown the cache misses into the three types of misses. What is the data cache miss rate?

Problem Statement

A system has a first-level data cache with the following setup:

- Size: 16 KB

- Associativity: Direct-mapped

- Line size: 64 bytes

- Addressing mode: Physical

- Separate instruction cache (no instruction cache misses to consider)

The following loop runs on the system:

for (i = 0; i < 4096; i++)

X[i] = X[i] * Y[i] + C;

- Arrays X and Y each have 4096 elements of 4 bytes (single-precision float), stored contiguously in physical memory.

- Below is the assembly output from a basic compiler:

loop:

lw f2, 0(r1)        # Load X[i]

lw f4, 0(r2)        # Load Y[i]

multd f2, f2, f4    # Multiply

addd f2, f2, f0     # Add C

sw 0(r1), f2        # Store X[i]

addi r1, r1, 4      # Increment pointer X

addi r2, r2, 4      # Increment pointer Y

addi r3, r3, 1      # Increment loop counter

bne r3, 4096, loop  # Loop until done

Solution

(a) Cache Miss Analysis of Given Code

- Cold Misses: Each array experiences a cold miss once every new cache line (64 bytes → 16 elements of 4 bytes). So, for 4096 elements:

  - Cold misses = 2 × (4096 / 16) = 512

- Conflict Misses:

  - All store operations to X[i] will miss because their corresponding line was overwritten by Y[i]'s load.

  - For each cache line loaded for Y, 15 of the 16 following elements will miss due to being evicted.

  - Conflict misses = 4096 (stores) + (15/16) × 4096 (loads to Y) = 7936

- Total Misses: 512 (cold) + 7936 (conflict) = 8448

- Total Data Accesses: 3 accesses per iteration (load X, load Y, store X) → 3 × 4096 = 12288

- Miss Rate: 8448 / 12288 = 0.6875 or 68.75%

(b) Software Optimization to Reduce Cache Misses

Option 1: Interleaving Arrays X and Y

- Arrange memory as: X[0], Y[0], X[1], Y[1], ...

- Now, each cache line holds 8 elements of X and 8 of Y.

- Cold misses = 4096 / 8 = 512

- Conflict misses = 0 (no overwrite between accesses)

- Total misses = 512

- Miss rate = 512 / 12288 = 0.0417 or 4.17%

Option 2: Separate X and Y by More than 16KB

- Prevents mapping to the same cache lines

- Cold misses: 2 × (4096 / 16) = 512

- Conflict misses = 0

- Total = 512, miss rate = 4.17%

(c) Hardware-Level Optimizations

Option 1: Increase Cache Size to 32KB

- With more cache, X and Y avoid evictions

- Cold misses = 512

- Conflict misses = 0

- Total misses = 512, miss rate = 4.17%

Option 2: Switch to Set-Associative Cache

- 2-way or 4-way associativity eliminates conflicting addresses

- Cold misses = 512

- Conflict misses = 0

- Total = 512, miss rate = 4.17%

Option 3: Increase Block Size by Factor of z

- Cold misses = 4096 / (16 × z)

- But increases conflict misses due to larger cache footprint

- Conflict misses = 2 × 4096 = 8192

- Not always helpful depending on z

Option 4: Implement Next-Line Prefetching

- Every cold miss also fetches the next cache line

- Cold misses = 2 × (4096 / 32) = 256

- Conflict misses:

  - 4096 store misses

  - (31/32) × 4096 = 3968 Y misses

- Total = 256 + 8064 = 8320

- Miss rate = 8320 / 12288 = 0.677 or 66.7%

Option 5: Add a Victim Cache

- Victim cache temporarily stores evicted lines

- Cold misses = 4096 / 16 = 256 (loads from X)

- Conflict misses = 4096 (loads from Y displacing X, but stores hit victim cache)

- Total = 256 + 4096 = 4352

- Miss rate = 4352 / 12288 = 0.3542 or 35.42%