# E1:

Problem

Consider the following MIPS code fragments, each containing two instructions. For each code fragment identify the type of hazard that exists between the two instructions and the registers involved.

a.

LD      R1, 0(R2)

DADD    R3, R1, R2

b.

MULT    R1, R2, R3

DADD    R1, R2, R3

c.

MULT    R1, R2, R3

MULT    R4, R5, R6

d.

DADD    R1, R2, R3

SD      2000(R0), R1

e.

DADD    R1, R2, R3

SD      2000(R1), R4

Hazard Analysis:

- a. Read After Write (RAW): The DADD instruction depends on the result of the LD instruction as it uses the value loaded into R1.

- b. Write After Write (WAW): Both MULT and DADD try to write to R1, leading to a WAW hazard.

- c. Structural Hazard: Two MULT instructions are issued in parallel, which may conflict if there is only a single multiplier unit.

- d. RAW Hazard: The SD instruction requires the value from the DADD instruction in R1 during its memory stage.

- e. RAW Hazard: The SD instruction depends on R1, which is written by the DADD instruction, and uses it during its execution (ALU) stage.

### E2:

Question a:
Describe how a 2-bit saturating counter branch predictor works. Illustrate the four states and explain the transition between them based on whether the branch is taken or not.

Answer:
A 2-bit saturating counter has four states:

- 00 (Strongly Not Taken)

- 01 (Weakly Not Taken)

- 10 (Weakly Taken)

- 11 (Strongly Taken)

Transitions:

- If the branch is taken:

  - 00 → 01

  - 01 → 10

  - 10 → 11

  - 11 → stays at 11

- If the branch is not taken:

  - 11 → 10

  - 10 → 01

  - 01 → 00

  - 00 → stays at 00

Question b:
Given the following C-style loop:

c

for (i = 0; i < N; i++) {

if (x[i] == 0)

y[i] = 0.0;

else

y[i] = y[i] / x[i];

}

And the equivalent assembly code:

loop:     L.D     F1, 0(R2)

L.D     F2, 0(R3)

BNEZ    F1, else

ADD.D   F2, F0, F0

BEZ     R0, fall

else:     DIV.D   F2, F2, F1

fall:     DADDI   R2, R2, 8

DADDI   R3, R3, 8

DSUBI   R1, R1, 1

S.D     -8(R3), F2

BNEZ    R1, loop

Assumptions:

- R1 holds N (the loop counter),

- R2 and R3 hold the base addresses of x and y, respectively,

- F0 is preloaded with 0.0,

- R0 always equals 0.

Prediction Scenario:
Assuming x contains alternating zeros starting from index 0 (i.e., x[0] = 0, x[1] ≠ 0, x[2] = 0, etc.), and the 2-bit predictor starts at state 00 (Strongly Not Taken).

We analyze predictions for the instruction: BNEZ F1, else.

Branch Behavior (First Few Iterations):

- Iteration 1 (x[0] == 0): Branch is not taken → Predictor remains at 00.

- Iteration 2 (x[1] ≠ 0): Branch is taken → Predictor moves to 01.

- Iteration 3 (x[2] == 0): Branch is not taken → Predictor returns to 00.

- Iteration 4 (x[3] ≠ 0): Branch is taken → Predictor moves to 01.

Solution

- 2-bit saturating counter branch predictor

| current counter value | prediction | actual outcome | new counter value |
| --- | --- | --- | --- |
| 00 | NT | NT | 00 |
| 00 | NT | T | 01 |
| 01 | NT | NT | 00 |
| 01 | NT | T | 10 |
| 10 | T | NT | 01 |
| 10 | T | T | 11 |
| 11 | T | NT | 10 |
| 11 | T | T | 11 |

- 2-bit counter prediction rate

| Iteration | current counter value | prediction | actual outcome | new counter value |
| --- | --- | --- | --- | --- |
| 1 | 00 | NT | NT | 00 (hit) |
| 2 | 00 | NT | T | 01 (miss) |
| 3 | 01 | NT | NT | 00 (hit) |
| 4 | 00 | NT | T | 01 (miss) |
| ... | ... | ... | ... | ... |